Attachment #1:

Drummond Company, Inc. (DCI) and the City of Jasper have a Memorandum of Understanding for a real estate transaction for property owned by DCI at its currently permitted Surface Mine No. 1.  The purpose of the transaction is for the development and operation of a recreational area, a Multiuse Facility/Sports Complex, by the City.  The current postmining landuse of Surface Mine No. 1 is Industrial/Commercial Recreation, as approved in Revision R-4.

Increment No. 1 currently has an existing highwall approximately 4,400 feet in length with an average depth of 99 feet.  However, 1,900 feet of the highwall is a pre-law highwall that has not been disturbed by DCI under this permit.  Therefore, DCI is requesting that the pre-law 1,900 feet of highwall be release as undisturbed.  This will result in a total of 2,500 feet of highwall with an average height of highwall of 99 feet and a maximum height of 190 feet.

DCI is proposing to change the Reclamation Plan within Increment No. 1 to grade the site and backfill the existing highwall to a grade and level to accommodate the construction of the Multiuse Facility/Sports Complex.  A proposed grading plan is included.  As part of the grading plan, it is requested to leave a portion of one section of highwall on the south end of the property, approximately 600 feet in length, with a maximum height of 80 feet and an average height of 54 feet and a portion of a section of highwall on the north end of the property, approximately 700 feet in length, with a maximum height of 120 feet and an average height of 80 feet. The remaining 1,200 feet of highway will be completely eliminated during the construction of the Multiuse Facility/Sports Complex.  The resulting length of highwall will be reduced from 2,500 feet to 1,300 feet with a maximum height of 120 feet and an average height of 51 feet.

The two remaining sections of highwall will be scaled using mobile equipment to provide safety and stability.  Drainage control measures and security fencing will be installed above and below the remaining highwalls to further provide safety and future stability.

The City of Jasper is requesting that these two sections of highwall be left for aesthetics and as an educational showcase teaching the mining history of Walker County.

See Attached Grading Plan.